# Automated Irrigation Controller Circuit Documentation

## 1. Circuit Summary

This circuit uses an ESP32-S3 to monitor a SparkFun soil moisture sensor and control a 5 V relay module.

- The soil moisture sensor is powered from the ESP32-S3 3.3 V and GND pins.

- The sensor’s analog signal is connected to ESP32-S3 GPIO4.

- The ESP32-S3 controls the relay module through GPIO20.

- The relay module is powered from the ESP32-S3 5 V and GND connections.

- The relay’s normally open contact switches a separate 5 V supply to a red LED.

- The LED illuminates when the relay is energized.

- The relay’s normally closed contact is not connected.

- The supplied firmware refers to the switched load as a pump, but the documented circuit contains an LED rather than a pump. Therefore, the LED is the actual load shown in this design.

The controller periodically samples the soil sensor, determines whether the soil is dry using a configurable threshold and hysteresis band, and activates the relay when the dry condition is detected.

## 2. Component List

### 2.1 ESP32-S3

The ESP32-S3 is the primary controller for the circuit.

Functions:

- Supplies 3.3 V power to the soil moisture sensor.

- Reads the sensor’s analog output on GPIO4.

- Controls the relay module through GPIO20.

- Executes the moisture monitoring and relay-control firmware.

- Provides 5 V and GND connections for the relay module.

### 2.2 SparkFun Soil Moisture Sensor

The soil moisture sensor provides an analog voltage corresponding to the detected soil moisture level.

Functions:

- Receives power from the ESP32-S3 3.3 V output.

- Connects its ground to the ESP32-S3 ground.

- Sends its analog measurement signal to ESP32-S3 GPIO4.

The sensor reading must be calibrated in the software because the actual analog values depend on the sensor, soil composition, moisture level, and installation conditions.

### 2.3 1 Channel 5 V Relay Module

The relay module provides electrically isolated switching contacts for the external 5 V LED circuit.

Functions:

- Receives 5 V power from the ESP32-S3 5 V pin.

- Receives a control signal from ESP32-S3 GPIO20.

- Uses its normally open contact to switch the LED circuit.

- Uses its common contact as the switched-supply connection.

- Has an unused normally closed contact.

The relay module’s control input polarity depends on the module design. The supplied code is configured for an active-high relay by default.

### 2.4 5 V DC Power Supply

The separate 5 V DC power supply powers the LED switching circuit through the relay contacts.

Functions:

- Supplies positive voltage to the relay COM contact.

- Supplies the negative return connection to the LED cathode.

- Provides power to the LED only when the relay contact is closed.

The power supply is used on the relay contact side and is not directly connected to the ESP32-S3 ground in the supplied net list.

### 2.5 LED Two Pin (Red)

The red LED provides a visual indication that the relay’s normally open contact has closed.

Functions:

- Its anode connects through the current-limiting resistor to the relay’s normally open contact.

- Its cathode connects to the negative terminal of the separate 5 V power supply.

- It illuminates when the relay is energized and the switched circuit is complete.

### 2.6 LED Current-Limiting Resistor

This resistor limits current through the red LED.

The component label identifies it as a 330-ohm resistor, while the component properties specify a resistance of 200 ohms. The electrical behavior and LED current depend on the actual installed resistance, so the physical resistor value should be verified.

Functions:

- Connects between the LED anode and the relay’s normally open contact.

- Limits LED current to help prevent damage to the LED.

For a 5 V supply and a typical red LED forward voltage of approximately 2 V:

- With 200 ohms, the approximate current is 15 mA.

- With 330 ohms, the approximate current is 9 mA.

A 330-ohm resistor is generally a conservative choice for an indicator LED.

## 3. Component Wiring Details

### 3.1 ESP32-S3 Wiring

| ESP32-S3 Pin | Connected To | Function |
| --- | --- | --- |
| 3V3 | Soil moisture sensor VCC | Powers the sensor |
| GND | Soil moisture sensor GND | Sensor ground return |
| GPIO4 | Soil moisture sensor SIG | Analog moisture measurement |
| GPIO20 | Relay module IN | Relay control output |
| GND | Relay module VCC- (GND) | Relay control ground |
| 5V | Relay module VCC+ | Relay module supply voltage |

Additional notes:

- GPIO4 is configured as an analog input.

- GPIO20 is configured as a digital output.

- The firmware initially turns the relay off during startup.

- The ESP32-S3 ground is shared with the soil sensor and relay module control ground.

### 3.2 SparkFun Soil Moisture Sensor Wiring

| Sensor Pin | Connected To | Function |
| --- | --- | --- |
| VCC | ESP32-S3 3V3 | Sensor power |
| GND | ESP32-S3 GND | Sensor ground |
| SIG | ESP32-S3 GPIO4 | Analog sensor output |

The sensor signal is sampled ten times by the firmware, and the samples are averaged to reduce measurement noise.

### 3.3 1 Channel 5 V Relay Module Wiring

#### Relay Module Control and Power Connections

| Relay Module Pin | Connected To | Function |
| --- | --- | --- |
| VCC+ | ESP32-S3 5V | Relay module positive supply |
| VCC- (GND) | ESP32-S3 GND | Relay module ground |
| IN | ESP32-S3 GPIO20 | Relay control signal |

#### Relay Contact Connections

| Relay Contact | Connected To | Function |
| --- | --- | --- |
| COM | 5 V power supply Positive | Supplies voltage to the switched contact |
| N.O. | Resistor pin1 | Provides switched positive voltage to the LED circuit |
| N.C. | Not connected | Unused normally closed contact |

The relay contact circuit operates as follows:

- The separate 5 V supply positive terminal is connected to COM.

- When the relay is off, COM and N.O. are open, so the LED remains off.

- When the relay is energized, COM and N.O. close.

- Current flows from the 5 V supply, through the relay contact, resistor, and LED, to the supply negative terminal.

### 3.4 5 V DC Power Supply Wiring

| Power Supply Pin | Connected To | Function |
| --- | --- | --- |
| Positive | Relay module COM | Supplies the switched LED circuit |
| Negative | LED cathode | Completes the LED return path |

The 5 V supply powers the LED load through the relay contacts. Its positive and negative terminals should not be reversed.

### 3.5 Red LED Wiring

| LED Pin | Connected To | Function |
| --- | --- | --- |
| Anode | Resistor pin2 | Receives current through the resistor |
| Cathode | 5 V power supply Negative | LED return path |

The LED must be installed with the correct polarity:

- The anode is the positive-side connection.

- The cathode is the negative-side connection.

- Reverse polarity prevents normal illumination and may damage the LED if excessive reverse voltage is applied.

### 3.6 LED Current-Limiting Resistor Wiring

| Resistor Pin | Connected To | Function |
| --- | --- | --- |
| pin1 | Relay module N.O. | Receives switched voltage |
| pin2 | LED anode | Feeds current-limited LED current |

The resistor is in series with the LED and must remain in the circuit. The design data contains a value discrepancy: the component is labeled “330 ohm,” while its listed property is “200 Ohms.” The installed value should be confirmed before operation.

## 4. Electrical Operation

### 4.1 Moisture Measurement

The ESP32-S3 supplies 3.3 V to the soil moisture sensor. The sensor produces an analog voltage on its SIG output, which is read by GPIO4.

The firmware:

- Reads GPIO4 ten times.

- Waits 5 ms between readings.

- Calculates the average value.

- Compares the average with the configured dry-soil threshold.

The current software assumes that a higher sensor value indicates drier soil:

constexpr bool DRY_WHEN_ABOVE = true;

If the installed sensor produces lower values when dry, this setting must be changed to false.

### 4.2 Relay Control

GPIO20 controls the relay module input.

The code is currently configured for an active-high relay:

constexpr bool RELAY_ACTIVE_LOW = false;

With this setting:

- A HIGH signal energizes the relay.

- A LOW signal de-energizes the relay.

If the relay module activates when its input is LOW, change the setting to:

constexpr bool RELAY_ACTIVE_LOW = true;

### 4.3 LED Indication

The LED is connected to the relay’s normally open contact. It therefore indicates that the relay has been energized.

Expected behavior:

| Soil Condition | Relay | N.O. Contact | LED |
| --- | --- | --- | --- |
| Wet | Off | Open | Off |
| Dry | On | Closed | On |

The firmware variable and comments refer to the controlled output as a pump. In this hardware design, the relay is switching the LED circuit instead.

## 5. Documented Firmware

### 5.1 Firmware Purpose

The firmware performs the following functions:

- Initializes the soil sensor input.

- Initializes the relay control output.

- Ensures the relay is off during startup.

- Samples and averages the soil moisture signal.

- Determines whether the soil is dry.

- Applies hysteresis to reduce relay chatter near the threshold.

- Controls the relay.

- Reports sensor readings and relay status through the serial port.

### 5.2 Complete Firmware

// Automated irrigation controller
// ESP32-S3 + SparkFun soil moisture sensor + 5 V relay
// Soil SIG -> GPIO4 (ADC), relay IN -> GPIO20.

#include <Arduino.h>

constexpr uint8_t SOIL_PIN = 4;
constexpr uint8_t RELAY_PIN = 20;

// Adjust these after observing the values printed in Serial Monitor.
// For this sensor, set DRY_WHEN_ABOVE according to your calibration.
constexpr int DRY_THRESHOLD = 2000;
constexpr int HYSTERESIS = 100;
constexpr bool DRY_WHEN_ABOVE = true;

// Many relay modules are active LOW. Change to false if your module is active HIGH.
constexpr bool RELAY_ACTIVE_LOW = false;

bool pumpOn = false;

void setPump(bool on) {
  pumpOn = on;

  const bool relayLevel = RELAY_ACTIVE_LOW ? !on : on;
  digitalWrite(RELAY_PIN, relayLevel ? HIGH : LOW);
}

int readSoil() {
  constexpr uint8_t samples = 10;
  uint32_t total = 0;

  for (uint8_t i = 0; i < samples; ++i) {
    total += static_cast<uint32_t>(analogRead(SOIL_PIN));
    delay(5);
  }

  return static_cast<int>(total / samples);
}

bool dryCondition(int value) {
  if (DRY_WHEN_ABOVE) {
    if (pumpOn) {
      return value > (DRY_THRESHOLD - HYSTERESIS);
    }

    return value >= (DRY_THRESHOLD + HYSTERESIS);
  }

  if (pumpOn) {
    return value < (DRY_THRESHOLD + HYSTERESIS);
  }

  return value <= (DRY_THRESHOLD - HYSTERESIS);
}

void setup() {
  Serial.begin(115200);

  pinMode(SOIL_PIN, INPUT);
  pinMode(RELAY_PIN, OUTPUT);

  setPump(false);  // Keep the relay off during startup.

  Serial.println("Irrigation controller started");
}

void loop() {
  const int moistureValue = readSoil();
  const bool soilIsDry = dryCondition(moistureValue);

  setPump(soilIsDry);

  Serial.print("Soil value: ");
  Serial.print(moistureValue);

  Serial.print(" | Status: ");
  Serial.print(soilIsDry ? "DRY" : "WET");

  Serial.print(" | Pump: ");
  Serial.println(pumpOn ? "ON" : "OFF");

  delay(1000);
}

### 5.3 Firmware Configuration

#### Soil Sensor Pin

constexpr uint8_t SOIL_PIN = 4;

The soil sensor signal is connected to GPIO4.

#### Relay Control Pin

constexpr uint8_t RELAY_PIN = 20;

The relay input is connected to GPIO20.

#### Dryness Threshold

constexpr int DRY_THRESHOLD = 2000;

The threshold should be adjusted based on actual sensor readings. Observe the values in the Serial Monitor under known wet and dry conditions.

#### Hysteresis

constexpr int HYSTERESIS = 100;

The hysteresis prevents the relay from rapidly switching when the sensor value fluctuates near the threshold.

With the default settings and DRY_WHEN_ABOVE = true:

- The relay turns on at or above 2100.

- Once on, it remains on while the reading is above 1900.

- The relay turns off at or below 1900.

#### Sensor Polarity

constexpr bool DRY_WHEN_ABOVE = true;

Set this to:

- true if higher readings indicate drier soil.

- false if lower readings indicate drier soil.

#### Relay Polarity

constexpr bool RELAY_ACTIVE_LOW = false;

Set this to:

- false for an active-high relay input.

- true for an active-low relay input.

## 6. Commissioning and Safety Notes

- Confirm the actual resistor value before powering the LED circuit.

- Verify the LED polarity before applying power.

- Confirm the relay module’s input polarity before relying on the software setting.

- Test the relay with the LED circuit disconnected or limited during initial firmware testing.

- Do not connect a pump or other high-current load directly to the ESP32-S3.

- If the relay is later used to control a mains-powered device, use an appropriately rated enclosure, fuse, wiring, and relay contact rating, and follow applicable electrical safety requirements.

- Calibrate the soil sensor in the actual soil and installation environment.

- Confirm that the ESP32-S3 5 V output and the selected power source can supply the relay module’s coil current.

- The relay contact-side 5 V supply is separate from the ESP32-S3 logic supply in the supplied wiring description.

- The unused relay N.C. contact should remain insulated and protected against accidental contact.